August 2022

| Americas Bosch Security Systems, Inc. 130 Perinton Parkway Fairport, New York, 14450, USA Phone: + 1 800 289 0096 Fax: +1 585 223 9180 security.sales@us.bosch.com www.boschsecurity.us | Europe, Middle East, Africa Bosch Security Systems B.V. P.O. Box 80002 5600 JB Eindhoven, The Netherlands Phone: + 31 40 2577 284 Fax: +31 40 2577 330 emea.securitysystems@bosch.com www.boschsecurity.com | Asia-Pacific Robert Bosch (SEA) Pte Ltd, Security Systems 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2600 Fax: +65 6571 2698 apr.securitysystems@bosch.com www.boschsecurity.com |
| --- | --- | --- |

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2004 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

SECTION 28 23 29

VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

BOSCH FLEXIDOME inteox 7100i

- – GENERAL

  - SUMMARY

    - Related Sections

      - Section [28 23 13 – Video Surveillance Control and Management Systems].

      - Section [28 23 16 – Video Surveillance Monitoring and Supervisory Interfaces].

      - Section [28 23 19 – Digital Video Recorders and Analog Recording Devices].

      - Section [28 23 23 – Video Surveillance Systems Infrastructure].

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

    - EMC – Emissions

      - EN 50121-4		Railway applications - Electromagnetic compatibility - Part 4: Emission and immunity of the signalling and telecommunications apparatus

      - EN 55032 (class B)	Electromagnetic compatibility of multimedia equipment - Emission requirements

      - 47CFR15, class B (STP)	Code of Federal Title 47 – Telecommunication Chapter I - FEDERAL COMMUNICATIONS COMMISSION, Subchapter A – GENERAL, Part 15 - RADIO FREQUENCY DEVICES

    - EMC – Immunity

      - EN 50121-4		Railway applications - Electromagnetic compatibility - Part 4: Emission and immunity of the signalling and telecommunications apparatus

      - EN 50130-4		Alarm systems - Part 4: Electromagnetic compatibility - Product family standard: Immunity requirements for components of fire, intruder, hold up, CCTV, access control and social alarm systems

    - Environmental

      - EN 50130-5 (Class IV)	Alarm systems - Part 5: Environmental test methods

      - EN 50581 (RoHS)	Technical documentation for the assessment of electrical and electronic products with respect to the restriction of hazardous substances

      - RoHS EU, 2011/65/EU and 2015/863/EU	Directive of the European Parliament and of the Council as regards the list of restricted substances

      - WEEE EU, 2012/19/EU	Directive of the European Parliament and of the Council of 4 July 2012 on waste electrical and electronic equipment (WEEE)

      - Packaging EU, 94/62/EU	European Parliament and Council Directive 94/62/EC of 20 December 1994 on packaging and packaging waste

      - N2580-1 (Bosch standard)	Prohibition and declaration of substances

    - Safety

      - EN 62368-1	Audio/video, information and communication technology equipment - Part 1: Safety requirements

      - UL 62368-1, 2nd Edition, 2014-12-01 (Audio/video, Information and Communication Technology Equipment - Part 1: Safety Requirements)

      - IEC 62368-1	Audio/video, information and communication technology equipment - Part 1: Safety requirements

      - EN 62471 (Eye safety for IR) Photobiological safety of lamps and lamp systems

    - ONVIF conformance

      - EN 50132-5-2	Alarm systems - CCTV surveillance systems for use in security applications - Part 5-2: IP Video Transmission Protocols

      - EN 62676-2	Video surveillance systems for use in security applications

    - Impact protection

      - EN 62262 (IK10)	Degrees of protection provided by enclosures for electrical equipment against external mechanical impacts (IK code)

    - Water/Dust protection

      - EN 60529 (IP66)	Degrees of protection provided by enclosures (IP Code)

      - UL50E (Type 4X)	Enclosures for Electrical Equipment, Environmental Considerations 1st edition

    - Marks

      - CE, FCC, WEEE, cULus, C-Tick, VCCI

  - SYSTEM DESCRIPTION

    - Section Includes

      - Video Surveillance Remote Devices

    - Performance Requirements

      - The fixed network camera shall be a full-featured fixed dome designed for discrete video surveillance applications in indoor and outdoor environments.

      - The fixed network camera shall be a high performance 1/1.8 inch CMOS sensor type with 8MP resolution.

      - The fixed network camera shall offer enhanced system flexibility with dual recording (iSCSI and microSD card) options.

      - The fixed network camera shall support the following dual, redundant power options:

        - Input options:

          - PoE (IEEE 802.3at, type 2, class 4)

          - 24 VAC

        - The fixed network camera shall default to use PoE power.

        - The fixed network camera shall switch with no interruption on camera operation to the 24 VAC power supply if power from the PoE power supply is lost.

      - The fixed network camera shall offer a High Dynamic Range of 87 dB for clear images in extreme high-contrast environments.

      - The fixed network camera shall provide direct network connection using H.265, H.264 and M-JPEG compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The fixed network camera shall offer embedded Intelligent Video Analytics (IVA) that eliminates dedicated PCs and associated software maintenance.

      - The fixed network camera shall conform to the ONVIF standard to provide interoperability with other conformant systems.

      - The fixed network camera shall offer configurable multi streaming with individually configurable 8 MP streams.

      - The fixed network camera shall offer bi-directional audio input/output.

      - The fixed network camera shall:

        - Offer IP66 environmental protection.

        - Offer IK10 impact resistance casing and dome.

        - Conform to the UL50E type 4X standard.

        - Support a temperature range of -40 ºC to +50 ºC (-40 ºF to +122 ºF) for continuous operation.

      - The fixed network camera housing shall be an aluminum, rugged design.

  - SUBMITTALS

    - Submit under provisions of Section [01 33 00.]

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Shop Drawings; include

      - System device locations on architectural floor plans.

      - Full Schematic of system, including wiring information for all devices.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - System device locations on architectural floor plans.

      - Wiring and connection diagram.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [20] years experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - Listed by cULus.

      - Complies to FCC and CE product specific requirements  Test methods are in accordance to international standards.  Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements of Section 01 60 00.

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

    - Bosch provides off-the-shelf availability for our top selling products and same-day or 24-hour shipping.

  - WARRANTY

    - Provide manufacturer’s warranty covering 3 years for replacement and repair of defective equipment.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support from 8:00 a.m. to 8:00 p.m. via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

[Bosch Security Systems, Inc.

130 Perinton Parkway

Fairport, New York, 14450, USA

Phone: + 1 800 289 0096

Fax: + 1 585 223 9180

security.sales@us.bosch.com

www.boschsecurity.us]

[Bosch Security Systems B.V.

P.O. Box 80002

5600 JB Eindhoven, The Netherlands

Phone: + 31 40 2577 284

Fax: +31 40 2577 330

emea.securitysystems@bosch.com

www.boschsecurity.com]

[Asia-Pacific

Robert Bosch (SEA) Pte Ltd, Security Systems

11 Bishan Street 21

Singapore 573943

Phone: +65 6571 2600

Fax: +65 6571 2698

apr.securitysystems@bosch.com

www.boschsecurity.com]

    - Substitutions: [Not permitted.] [Under provisions of Division 1.]

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - BOSCH NDE-7604-AL, NDE-7604-AL-OC FLEXIDOME inteox 7100i camera or comparable

    - General Characteristics:

      - The fixed network camera shall provide a 1/1.8-inch CMOS image sensor with the following minimum capabilities:

        - 3840x2160 (8 MP) effective picture elements.

        - Sensitivity down to 0.189 lux (color) and 0.0316 lux (mono)

      - The fixed network camera shall offer an Automatic Varifocal (AVF) lens with P-iris control and a focal length of 3.6 to 10 mm.

      - The fixed network camera shall provide direct network connection using H.265, H.264 and M-JPEG compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The fixed network camera shall offer a High Dynamic Range of 87 dB for clear images in extreme high-contrast environments.

      - The fixed network camera shall offer advanced Intelligent Video Analytics (IVA) with automated calibration by use of a gyro-sensor.

      - The fixed network camera shall offer bi-directional audio input/output.

      - The fixed network camera shall offer the ability to define a rectangular mosaic mask.

      - The fixed network camera shall support dynamic privacy masking of moving persons.

      - The fixed network camera shall support the following redundant power options:

        - Input options:

          - PoE (IEEE 802.3at, type 2, class 4)

          - 24 VAC

        - The fixed network camera shall default to use PoE power.

        - The fixed network camera shall switch with no interruption on camera operation to the 24 VAC power supply if power from the PoE power supply is lost.

      - The fixed network camera shall be able to be direct surface mounted to a wall or ceiling.

      - The fixed network camera shall be capable of operating in an indoor and outdoor environment.

      - The fixed network camera shall:

        - Offer IP66 environmental protection.

        - Offer IK10 impact resistance casing and dome.

        - Conform to the UL50E type 4X standard.

        - Support a temperature range of -40 ºC to +50 ºC (-40 ºF to +122 ºF) for continuous operation.

      - The fixed network camera housing shall be an aluminum, rugged design.

      - The fixed network camera shall support the following languages:

        - English

        - Czech

        - Dutch

        - French

        - German

        - Italian

        - Polish

        - Portuguese

        - Russian

        - Spanish

        - Japanese

        - Chinese

    - Imaging

      - The fixed network camera shall offer a 1/1.8-inch CMOS image sensor.

      - The fixed network camera shall offer a minimum effective number of pixels of 3840x2160 (8MP).

      - The fixed network camera shall offer a 16:9 aspect ratio.

      - The fixed network camera shall produce a color image with a minimum scene illumination of 0.189 lux and a monochrome image, when in the night mode, with a minimum illumination of 0.0316 lux at 30 IRE (3.6-10 mm lens), measured according to IEC 62676 Part 5 (1/25, F1.5).

      - The fixed network camera shall offer one push auto focus and P-iris control with manual override.

      - The fixed network camera shall offer a High Dynamic Range (HDR) of 87 dB.

    - Image Processing

      - The fixed network camera shall support dynamic noise reduction to reduce bandwidth and storage requirements by optimizing the detail-to-bandwidth ratio via temporal and spatial noise filtering.

      - The fixed network camera shall include intelligent streaming functionality to reduce bandwidth and storage requirements by optimizing the camera encoder on to camera noise level.

      - The fixed network camera shall be capable of capturing and storing images using H.265 and H.264 compression at 8 MP resolution.

      - The fixed network camera shall deliver 8 MP video, at rates up to 30 images per second.

      - The camera shall allow regions of interest to be sent in separate streams, so it is possible to view both an overview and a detail at the same time.

    - System Features

      - The fixed network camera shall be compatible with the Bosch Video Client, Video Security client and the Bosch Video Management System.

      - The fixed network camera shall provide one (1) audio mono line in and one (1) audio mono line out.

      - The fixed network camera shall provide one (1) alarm input and two (2) alarm outputs.

    - PTRZ Features

      - The fixed network camera shall offer an Automatic Varifocal (AVF) lens with P-iris control and a focal length of 3.6 to 10 mm.

      - The fixed network camera shall have a field of view of 97° to 46°.

      - The fixed network camera shall provide a pan range of 0º to 370º.

      - The fixed network camera shall provide a tilt range of 0º to 75º.

      - The fixed network camera shall provide a roll angle of 0º to 355º.

    - Pre-programmed Modes

      - The fixed network camera shall allow users to customize these modes for the specific requirements of the camera site.

    - Recording and Storage Management

      - The fixed network camera shall have a microSD card slot that uses standard, off-the-shelf SD (Secure Digital), SDHC (Standard Digital High Capacity), or a SDXC (Secure Digital eXtended Capacity) card for local storage (up to 2 TB).

      - The fixed network camera shall support industrial SD cards.

      - The local storage feature shall be capable of storage for Automatic Network Replenishment (ANR).

      - The fixed network camera shall offer enhanced system flexibility with dual recording (iSCSI and microSD card) options.

      - The fixed network camera shall support iSCSI devices to allow video stream to be recorded directly to an iSCSI RAID array.

      - The fixed network camera shall support iSCSI storage targets.

      - The fixed network camera shall be compatible with the Bosch Video Recording Manager (VRM) to control and manage video recording.

    - Resolution Characteristics

      - The fixed network camera shall generate 8MP resolution using H.265 compression (ISO/IEC 14496-10).

      - The fixed network camera shall generate multiple simultaneous configurable 8 MP video streams.

      - The fixed network camera shall allow simultaneous streaming of individual 8 MP streams, and allow a choice of 8MP resolution in combination with HD resolutions.

    - IP Connectivity

      - The fixed network camera shall support iSCSI devices to allow the network-enabled camera to stream video directly to an iSCSI RAID array.

      - The fixed network camera shall conform to the ONVIF Profile S, G and T standard.

      - The fixed network camera shall offer Quality of Service (QoS) configuration options.

      - The fixed network camera shall support the IPv6 internet-layer protocol for packet switched internetworking across multiple IP networks.

    - Intelligent Video Analytics (IVA)

      - The fixed network camera shall offer advanced embedded Intelligent Video Analytics (IVA) that eliminates dedicated PCs and associated software maintenance.

      - The fixed network camera shall offer embedded machine learning capabilities that eliminates dedicated PCs and associated software maintenance.

      - The camera shall use machine learning to allow users to define and classify objects of interest, both moving and non-moving, and then generate detectors for the objects. The machine learning program shall allow distribution of detectors to other cameras.

      - The camera shall include AI-based vehicle detection for traffic monitoring, with the following characteristics:

        - AI vehicle detector shall not require calibration.

        - Vehicle headlights and shadows shall not trigger alerts and counts.

        - The detector shall be able to separate vehicles even in dense congestion or traffic queues.

        - AI vehicle detector shall be configurable to analyze different scenes for one or more events including:  traffic congestion, wrong-way vehicle, presence and volume of vehicles, automatic incident detection.

        - AI detector shall collect traffic statistics.

        - AI detector shall detect parking space vehicle occupancy status.

      - The fixed network camera shall offer advanced Intelligent Video Analytics (IVA) with automated calibration by use of an integrated gyro-sensor so only camera height needs to be set for a full calibration.

      - The fixed network camera shall offer advanced embedded Intelligent Video Analytics (IVA) with the following functionality:

        - Triggers: any object, object in field, line crossing, enter field, leave field, loitering, follow route, idle object, removed object, counting, occupancy, crowd density, condition change, similarity search, flow, counter flow, conditional change

        - Filters: duration, size, aspect ratio, speed, direction, color, object classes

        - Tracking modes: standard (2D) tracking, 3D tracking, 3D people tracking, ship tracking, museum mode.

        - Tamper detection: global change, scene too bright, scene too dark, reference check

        - Face detection

      - The fixed network camera shall allow users to set up to ten (10) separate profiles and switch profiles based on a day/night or holiday schedules.

      - The fixed network camera shall support people counting.

      - The fixed network camera shall incorporate an IVA alarm rule engine that supports programming of alarm tasks which are individually reported to the video management software.

    - Access Security

      - The camera shall offer three levels of password protection.

      - The camera shall incorporate Trusted Platform Module (TPM) functionality provided by “Secure Element” (dedicated AES/DES/PKI crypto co-processor) for storage of encryption keys.

      - The camera shall have cryptography with a maximum of 4096-bit RSA keys for TLS (maximum version 1.3) and device identity.

      - The camera shall have advanced certificate handling with unique certificates and encrypted private keys.

      - The camera shall support secured connections.

      - The camera shall require a service-level password at initial setup to access camera functions.

      - The camera shall disable all unsecure ports.

      - The camera shall disable all unsecure remote communication.

      - The camera shall prevent uploading of untrusted, unauthenticated third-party software.

      - The camera shall support only authenticated firmware and third-party apps.

      - The camera shall include a sandbox environment for the secure execution of trusted third-party software.

      - The camera shall incorporate an embedded login firewall.

      - The camera shall have software protection that seals the camera configuration (“software sealing”) and detects changes to camera configuration.

    - Installation Requirements

      - The fixed network camera shall support the following redundant power options:

        - Input options:

          - PoE (IEEE 802.3at, type 2, class 4)

          - 24 VAC

        - The fixed network camera shall default to use PoE power.

        - The fixed network camera shall switch with no interruption on camera operation to the 24 VAC power supply if power from the PoE power supply is lost.

      - The fixed network camera shall be able to be direct surface mounted to a wall or ceiling.

      - The fixed network camera shall be capable of operating in an indoor and outdoor environment.

      - The fixed network camera shall:

        - Offer IP66 environmental protection.

        - Offer IK10 impact resistance casing and dome.

        - Conform to the UL50E type 4X standard.

        - Support a temperature range of -40 ºC to +50 ºC (-40 ºF to +122 ºF) for continuous operation.

      - The fixed network camera housing shall be an aluminum, rugged design.

      - The fixed network camera shall provide a multi-language on-screen display.

  - ACCESSORIES

    - Mounts & accessories

      - NDA-8000-PIP	Pendant interface plate, indoor

      - NDA-U-CMT 	Corner mount adapter

      - NDA-U-PMAL	Pole mount adapter large

      - NDA-U-PMAS	Pole mount adapter small

      - NDA-U-PMT		Pendant pipe mount, 12” (31 cm)

      - NDA-U-PMTE	Pendant pipe extension, 20” (50 cm)

      - NDA-U-PSMB	Pendant wall/ceiling mount SMB

      - NDA-U-RMT		Pendant parapet mount

      - NDA-U-WMP	Wall mount plate

      - NDA-U-WMT		Pendant wall mount

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMONSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION

202208310930