Bosch Security Systems is a leading global supplier of security, safety, and communications products and systems. Protecting lives, buildings, and assets is the major aim. The product portfolio includes video security, intrusion detection, and fire detection as well as access control and management systems.

For additional information, contact:

| Americas Bosch Security Systems, Inc. 130 Perinton Parkway Fairport, New York, 14450, USA Phone: + 1 800 289 0096 Fax: +1 585 223 9180 CSI@us.bosch.com www.boschsecurity.us | Europe, Middle East, Africa Bosch Security Systems B.V. P.O. Box 80002 5600 JB Eindhoven, The Netherlands Phone: + 31 40 2577 284 Fax: +31 40 2577 330 emea.securitysystems@bosch.com www.boschsecurity.com | Asia-Pacific Robert Bosch (SEA) Pte Ltd, Security Systems 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2600 Fax: +65 6571 2698 apr.securitysystems@bosch.com www.boschsecurity.com |
| --- | --- | --- |

Intelligent Video Analytics (IVA) Pro License Plate + Make Model

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Video Surveillance

28 21 00 	Surveillance Cameras

28 21 13	IP Cameras

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold bracketed text>.

2.  Explanatory notes and comments are presented in italicized colored text.

Intelligent Video Analytics (IVA) Pro License Plate + Make Model

- GENERAL

  - SUMMARY

    - Section includes Intelligent Video Analytics (IVA) Pro License Plate + Make Model, a video analytics software based on deep neural network technology for detecting license plate and vehicle information.

  - SUBMITTALS

    - Informational Submittals

      - Product Data

        - Manufacturer’s printed or electronic data sheets

        - Manufacturer’s installation and operation manuals

        - Warranty documentation

  - QUALIFICATIONS

    - Installers shall be certified, trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Bosch Security Systems, Inc.

130 Perinton Parkway

Fairport, New York, 14450

USA

Phone: +1 800 289 0096

Fax: +1 585 223 9180

Email: security.sales@us.bosch.com

www.boschsecurity.com

Bosch Security Systems, B.V.

P.O. Box 80002

5600 JB Eindhoven, The Netherlands

Phone: +31 40 2577 284

Fax: +31 40 2577 330

Email: emea.securitysystems@bosch.com

www.boschsecurity.com

Robert Bosch (SEA) Pte Ltd, Security Systems

11 Bishan Street 21

Singapore 573943

Phone: +65 6571 2600

Fax: +65 6571 2698

Email: apr.securitysystems@bosch.com

www.boschsecurity.com

    - Product Model:	MVC-IVA-LPRX

  - GENERAL DESCRIPTION

    - The video analytics software shall detect and read license plates on moving vehicles (car, van, bus, truck, motorbike). The license plate information shall be transmitted in real time via different protocols ensuring easy integration.

    - The video analytics software shall detect and recognize the make and model of moving vehicles (car, van, bus, truck). The make and model information shall be transmitted in real time via different protocols ensuring easy integration.

    - The video analytics software shall excel in real-time identification of license plates in parking applications or city environments. It shall support strategies that improve traffic flow, reduce congestion, and enhance safety at intersections, toll booths, and parking facilities.

    - The video analytics software shall excel in real-time recognition of vehicles in parking applications or city environments. It shall enable more refined forensic search for specific vehicles in places where license plate recognition is no option.

    - The video analytics software shall ensure reliable and precise data capture for automated ticketing systems, traffic enforcement, and smart parking solutions.

    - The video analytics software shall ensure reliable and precise data capture to ensure reliable search results in the video management software.

    - The video analytics software shall detect at a maximum speed of the vehicle of 120 km/h (75 mph).

    - The video analytics software shall detect at a maximum number of 2 road lanes. Best performance shall be at 1 road lane.

    - The video analytics software shall detect at a detection resolution of 1280 x 720 px.

    - The video analytics software shall have a minimum of required pixels of 16 px on character height of license plate.

    - The video analytics software shall recognize more than 140 manufacturers and more than 2000 different models.

    - The video analytics software shall provide section integration of ONVIF, and Bosch Metadata.

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.

  - INSTALLATION

    - The contractor shall follow all Manufacturer-published guidance on proper installation and configuration of the installed cameras.

    - The Contractor shall test the system in conditions simulating the final installed environment

    - Reports:

      - System test results

      - System network addressing and device settings

END OF SECTION

202411140854