Bosch Security Systems is a leading global supplier of security, safety, and communications products and systems. Protecting lives, buildings, and assets is the major aim. The product portfolio includes video security, intrusion detection, and fire detection as well as access control and management systems.

For additional information, contact:

| Americas Bosch Security Systems, Inc. 130 Perinton Parkway Fairport, New York, 14450, USA Phone: + 1 800 289 0096 Fax: +1 585 223 9180 CSI@us.bosch.com www.boschsecurity.us | Europe, Middle East, Africa Bosch Security Systems B.V. P.O. Box 80002 5600 JB Eindhoven, The Netherlands Phone: + 31 40 2577 284 Fax: +31 40 2577 330 emea.securitysystems@bosch.com www.boschsecurity.com | Asia-Pacific Robert Bosch (SEA) Pte Ltd, Security Systems 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2600 Fax: +65 6571 2698 apr.securitysystems@bosch.com www.boschsecurity.com |
| --- | --- | --- |

2MP OUTDOOR IR FIXED DOME CAMERA

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Video Surveillance

28 21 00 	Surveillance Cameras

28 21 13	IP Cameras

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold bracketed text>.

2.  Explanatory notes and comments are presented in italicized colored text.

2MP OUTDOOR IR FIXED DOME CAMERA

- GENERAL

  - SUMMARY

    - Section includes a 2 MP IP66/IP67-rated outdoor fixed dome camera with H.264 and H.265 encoding, dynamic range of 141 dB, and built-in artificial intelligence providing full situational awareness and triggers for relevant alerts.

## Related Requirements

      - 28 05 00 Common Work Results

      - 28 05 07.13 Power Sources for Video Surveillance

      - 28 05 07.21 PoE Power Sources for Electronic Safety and Security

      - 28 05 11 Cyber Security Requirements for Electronic Safety and Security

      - 28 05 19.13 Hybrid Digital Video Recorders

      - 28 05 19.15 Network Video Recorders

      - 28 05 33 Safety and Security Network Communications Equipment

      - 28 06 20 Schedules for Video Surveillance

      - 28 21 19 Camera Mounts

      - 28 23 00 Video Management System

      - 27 15 00 Communications Horizontal Cabling

      - 27 20 00 Data Communications

      - 28 16 15 Access Control Interfaces to Video Surveillance

      - 28 31 31.15 Intrusion Detection Interfaces to Video Surveillance

      - 28 51 15.13 Information Interfaces to Video Surveillance Systems

    - Sustainability

      - The fixed network camera shall be manufactured in compliance with ISO 9001 and ISO 14001.

      - The fixed network camera shall be compliant with the EU Directives 2011/65/EU (RoHS) and 2012/19/EU (WEEE).

      - Packaging shall be compliant with EU Directive 94/62/EC (amended by 2004/12/EC).

      - The fixed network camera shall be compliant with the EU regulation 1907/2006 (REACH).

  - REFERENCES

    - Abbreviations

      - AI – Artificial Intelligence

      - ARP – Address Resolution Protocol

      - DHCP - Dynamic Host Configuration Protocol

      - DNS - Domain Name Server

      - DDNS – Dynamic DNS

      - DORI - Detect, Observe, Recognize, Identify

      - fps - frames per second

      - HEVC – High Efficiency Video Coding

      - HTTPS – Secure Hypertext Transfer Protocol

      - ICMP - Internet Control Message Protocol

      - IP - Internet Protocol

      - IR – InfraRed

      - JPEG - Joint Photographic Experts Group

      - mDNS – Multicast DNS

      - MP - Megapixel

      - MJPEG - Motion JPEG

      - NTP - Network Time Protocol

      - PKI - Public Key Infrastructure

      - PoE - Power over Ethernet

      - RTCP – Real-Time Control Protocol

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - TCP - Transmission Control Protocol

      - TPM - Trusted Platform Module

      - TLS – Transport Layer Security

      - UDP - User Datagram Protocol

    - Reference Standards

      - Network

        - IEEE 802.3af/802.3at (Power over Ethernet)

        - IEEE 802.1x (EAP-TLS, PEAP-MSCHAPv2) (Authentication)

        - IPv4 (RFC 791)

        - IPv6 (RFC 2460)

        - QoS – DiffServ (RFC 2475)

        - NIST SP500-267

      - Cybersecurity

        - ETSI EN 303 645

        - IEC 62443-4-1

        - UL 2900-2-3

      - Video

        - ITU H.264

        - ISO/IEC 14496-10 Advanced Video Coding (H.264)

        - H.265/HEVC

        - ISO/IEC 23008-5 Advanced Video Coding (H.265)

        - MJPEG

        - SMPTE 296M (HDTV 720p)

        - SMPTE 274M (HDTV 1080p)

      - ONVIF

        - EN 50132-5-2

      - Emissions

        - FCC CFR 47 part 15 (Class A)

        - EN 55032 - Electromagnetic compatibility of multimedia equipment. Emission Requirements – Class A

        - EN 55035 - Electromagnetic compatibility of multimedia equipment. Emission Requirements – Class A

        - ICES-003, Class A

        - EN 50130-4

        - EN 50121-4 (Railway applications)

        - EN 55016-2-1

        - EN 61000-4-2

        - EN 61000-4-3

        - EN 61000-4-4

        - EN 61000-4-5

        - EN 61000-4-6

        - EN 61000-4-8

        - EN 61000-6-1

        - EN 61000-6-3

        - CISPR 32 (Class A)

        - CISPR 35 (Class A)

      - Safety

        - EN 62368-1 ed. 3

        - IEC 62368-1 ed. 3

        - UL 62368-1 ed. 3

        - IEC 60950-22

        - CAN/CSA-C22.2 No. 62368-1 ed. 3

        - IEC/EN 62471

        - IS 13252

      - Environmental

        - IEC

          - IEC 60068-2-1

          - IEC 60068-2-2

          - IEC 60068-2-5

          - IEC 60068-2-6

          - IEC 60068-2-14

          - IEC 60068-2-18

          - IEC 60068-2-27

          - IEC 60068-2-30

          - IEC 60068-2-42

          - IEC 60068-2-52

          - EN 60068-2-75

          - IEC 60068-2-78

          - EN 60529 – Degrees of protection provided by enclosures – IP66/IP67

          - ISO 20653 IP6K9K

          - UL50E Type 4X

          - IEC 62262 - Degrees of protection provided by enclosures for electrical equipment against external mechanical impacts – IK11

          - NEMA TS 2-2021 | Compliant to the next chapters when using a TS-2 compliant power supply: 2.1.1, 2.1.2, 2.1.3, 2.1.4, 2.1.5, 2.1.9 and 2.1.10

          - NEMA TS 2-2021 | Tested according to chapter 2.2.7, 2.2.8 and 2.2.9

          - ISO 21207 (Method B)

          - IEC 62236-4

  - SUBMITTALS

    - Informational Submittals

      - Product Data

        - Manufacturer’s printed or electronic data sheets

        - Manufacturer’s installation and operation manuals

        - Warranty documentation

      - Shop Drawings

      - Cyber hardening guidance

    - Closeout Submittals

      - Final listing of devices and settings

      - System test results

      - Statement of compliance with Manufacturer Cyber Hardening Guidelines

  - QUALIFICATIONS

    - Installers shall be certified, trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY AND SUPPORT

    - Manufacturer shall provide a limited 5-year warranty for the product to be free of defects in material and workmanship.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Bosch Security Systems, Inc.

130 Perinton Parkway

Fairport, New York, 14450

USA

Phone: +1 800 289 0096

Fax: +1 585 223 9180

Email: security.sales@us.bosch.com

www.boschsecurity.com

Bosch Security Systems, B.V.

P.O. Box 80002

5600 JB Eindhoven, The Netherlands

Phone: +31 40 2577 284

Fax: +31 40 2577 330

Email: emea.securitysystems@bosch.com

www.boschsecurity.com

Robert Bosch (SEA) Pte Ltd, Security Systems

11 Bishan Street 21

Singapore 573943

Phone: +65 6571 2600

Fax: +65 6571 2698

Email: apr.securitysystems@bosch.com

www.boschsecurity.com

    - Product Model:	NDE-8702-RXL

  - GENERAL DESCRIPTION

    - The camera shall be a 2 MP fixed dome type with integrated split IR illumination, motorized pan, tilt, roll and zoom, starlight X, HDR X, H.264 and H.265 encoding, dynamic range of 141 dB, and built-in artificial intelligence providing full situational awareness and triggers for relevant alerts.

    - The fixed dome camera shall possess the following primary characteristics:

      - H.264, H.265, and MJPEG compression

      - Resolution up to 1920 x 1080 pixels

      - Motorized pan, tilt, roll and zoom for remote adjustment

      - Built-in intelligent IR illumination up to 50m (164ft)

      - Starlight X and HDR X technology

      - Frame rate up to 60 fps

      - Suite of deep learning-based video analytics

      - Bi-directional audio

      - DORI technology to facilitate distinguishing of persons or objects

      - Multiple data security elements including Trusted Platform Module (TPM), PKI, 4096-bit RSA encryption, and update capability for future software security functions.

      - Available posting to cloud-based services

      - Pre-configured optimized scene modes

      - Remote camera management

      - Mobile device camera setup and configuration

      - IP66, IP67, UL Type 4X, and IK11 environmental ratings

      - Power:	24 VAC, 12 to 26 VDC, or PoE+ IEEE 802.3at Type 2, Class 4

      - The fixed network camera shall provide a 12 VDC 50 mA power output.

  - ADDITIONAL VIDEO/CAMERA SPECIFICATIONS

    - Imaging Device:

      - The fixed network camera shall offer a 1/1.8-inch CMOS image sensor.

      - The fixed network camera shall offer a maximum effective number of pixels of 1920x1080 (2.1MP).

    - Lens:

      - The fixed network camera shall offer an Automatic Varifocal (AVF) lens with P-iris and a focal length of 4.4 to 10 mm.

      - The fixed network camera shall offer a lens aperture of F/1.3 to F/1.97.

      - The fixed network camera shall have an horizontal field of view of 110° to 48°.

      - The fixed network camera shall offer automatic focus and P-iris control with manual override.

      - The fixed network camera shall provide motorized Pan/Tilt/Roll/Zoom (PTRZ) for remote (re-) commissioning.

      - The fixed network camera shall provide a pan range of 0º to 361º.

      - The fixed network camera shall provide a roll angle of -95º to 95º.

    - Video Streaming:

      - The fixed network camera shall provide direct network connection using H.265, H.264 and M-JPEG compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The fixed network camera shall provide one M-JPEG and at least three H.264/H.265 independently configurable streams.

      - The fixed network camera shall support the following video resolutions:

        - HD 1080p 1920 x 1080, 1.3MP 1536 x 864, 1.3MP (5:4) 1280 x 1024, HD 720p 1280 x 720, SD 432p 768 x 432, SD 480p (4:3) 720 x 480, VGA (4:3) 640 x 480

      - The fixed network camera shall have support for both 16:9 - wide screen and 9:16 - upright aspect ratios.

      - The fixed network camera shall deliver HD 1080p video, at rates up to 60 images per second.

      - The fixed network camera shall offer a High Dynamic Range of 141 dB for clear images in extreme high-contrast environments.

    - Additional Video Functions:

      - The fixed network camera shall support day/night modes with automatic adjustable switch points, allowing it to operate in both color and monochrome settings.

      - The fixed network camera shall include backlight compensation.

      - The fixed network camera shall offer video watermarking capabilities.

      - The fixed network camera shall provide display stamping.

      - The fixed network camera shall be capable of displaying its location information.

      - The fixed network camera shall feature electronic image stabilization to reduce the effects of camera shake.

      - The fixed network camera shall support a white balance range from 2000 K to 10,000 K.

      - The fixed network camera shall offer multiple white balance modes, including:

        - Basic

        - Standard

        - Dominant color

        - Manual mode

        - Hold mode

        - 4 automatic modes

        - Sodium lamp

      - The fixed network camera shall allow the definition of an area within the Field of View (FOV) as an exposure priority area, enhancing the visibility of objects in that specified area.

      - The fixed network camera shall offer the ability to define up to 8 privacy masks.

      - The fixed network camera shall offer multiple user defined privacy masks patterns, including:

        - Black

        - White

        - Gray

        - Auto color

        - 16M colors

        - Mosaic

        - 10 blurred levels

      - The fixed network camera shall produce a monochrome image with a minimum scene illumination of 0.0007 lux.

      - The fixed network camera shall produce a color image with a minimum scene illumination of 0.009 lux.

      - The fixed network camera shall offer intelligent IR illumination up to 50m (164ft) provided by a split design with 4 (four) IR LEDs.

      - The fixed network camera shall be capable of automatically switching on the relevant IR LEDs, depending on the lens position and field of view when using PTRZ functionality.

      - The fixed network camera shall allow for manual override to individually configure the IR intensity of each LED.

      - The fixed network camera shall offer HDR (High Dynamic Range) X to enhance image quality in both bright and dark areas.

      - The fixed network camera shall offer three types of HDR X:

        - Motion optimized

        - Extreme Dynamic Range

        - Optimized Dynamic Range

      - The fixed network camera shall offer starlight X technology to deliver superior image performance in low-light environments.

      - The fixed network camera shall support Intelligent Defog to automatically adjust the contrast of the image in foggy scenes.

      - The fixed network camera shall offer comprehensive video content analysis capabilities, including detection and classification of persons and vehicles.

      - The fixed network camera shall offer a wide range of video analytics rule configuration, including:

        - Any object

        - Object in field

        - Line crossing

        - Enter / leave field

        - Loitering

        - Follow route

        - Idle object

        - Removed object

        - Counting

        - Occupancy

        - Crowd density estimation

        - Condition change

        - Similarity search

        - Flow / counter flow

        - Objects starting to move

        - Objects stopping

        - Tampering

        - Rule based alarms and tracking

        - Audio detection (May also be used to generate an alarm.)

      - The fixed network camera shall be configurable to execute the following actions upon alarm activation:

        - Turn its built-in output on or off

        - Turn another supported camera output on the same network on or off

        - Stream video to a compatible decoder

        - Send an e-mail with an image attachment

    - Local SD storage

      - The fixed network camera shall have dual microSD card slots that use standard off-the-shelf SD (Secure Digital) up to 2 TB, SDHC (Standard Digital High Capacity) or SDXC (Secure Digital eXtended Capacity).

      - The fixed network camera shall support multiple recording modes, including manual, scheduled, pre-alarm and alarm recording.

      - The fixed network camera shall support multiple storage configurations, including:

        - Mirror (redundant storage)

        - Failover (extended service interval)

        - Extend (maximum retention time)

        - Automatic Network Replenishment.

      - The fixed network camera shall offer enhanced system flexibility with dual recording (iSCSI and SD card) options.

      - The fixed network camera shall support health monitoring on at least one microSD card with this capability.

      - The fixed network camera shall support uploading JPEG images with video analytics overlay to an FTP server.

    - Audio

      - The fixed network camera shall provide switchable audio input. This includes one (1) line-in level (10 kOhm typical, max. 1 Vrms) or one (1) microphone-in level (2.5 VDC, 4 mA).

      - The fixed network camera shall provide one (1) audio line out (1.5 kOhm typical, 1 Vrms).

      - The fixed network camera shall provide one (1) alarm input and one (1) alarm output.

      - The fixed network camera shall offer the G.711 8 kHz, L16 16 kHz and AAC compression rates

      - The fixed network camera shall provide full duplex and half duplex streaming.

      - The fixed network camera shall be capable of disabling audio input permanently with a license code.

    - ONVIF

      - The fixed network camera shall conform to the ONVIF Profile S, G, T and M.

    - Video Analytics

      - The fixed network camera shall offer embedded deep learning-based detection and video analytics (IVA Pro) that eliminate dedicated PCs and associated software maintenance.

      - The fixed network camera shall offer embedded IVA Pro Buildings, IVA Pro Perimeter and IVA Pro Privacy for reliable deep learning-based detection.

      - The fixed network camera shall support other license options, including:

        - IVA Pro Traffic

        - IVA Pro Appearance

        - IVA Pro Personal Protective Equipment

        - IVA Pro License Plate (available with a future FW release)

        - IVA Pro Vehicle Make Model (available with a future FW release)

        - IVA Pro Dangerous Goods Signs (available with a future FW release)

        - IVA Pro License Plate + Make Model (available with a future FW release)

        - IVA Pro Gun Detection (Visual & Audio – available with a future FW release)

      - The fixed network camera shall provide Camera Trainer to allow the user to train the system for custom object detection and classification.

      - The fixed network camera shall be capable of running 16 different video analytics rules simultaneously.

      - The fixed network camera shall support automatic switching of video analytics types according to a predefined schedule.

      - The fixed network camera shall be capable of generating metadata, including object speed, geolocation, height, width, and classification to be recorded with the video.

      - The fixed network camera shall be capable of detecting and classifying up to 64 objects.

      - The fixed network camera shall support machine learning to enable users to classify customized objects.

  - NETWORK

    - Connectivity:

      - The fixed network camera shall offer versatile network connectivity options, such as 10/100/1000BASE-T and Ethernet interfaces.

    - Protocols supported:

      - The fixed network camera shall support the following protocols:

        - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

        - Configuration: Dynamic Host Configuration Protocol (DHCP) and fixed IP address

        - Web services: Internet Control Message Protocol (ICMP) and ICMP v6, Hypertext Transfer Protocol (HTTP), Secure Hypertext Transfer Protocol (HTTPS)

        - Network services: Address Resolution Protocol (ARP), Domain Name System (DNS), Network Time Protocol (NTP), File Transfer Protocol (FTP), System Network Management Protocol (SNMP) v3 and MIBII

        - Media: Real-Time Control Protocol (RTCP), Real-Time Transport Protocol (RTP), Real-Time Streaming Protocol (RTSP); Internet Group Messaging Protocol (IGMP), DiffServ (QoS)

        - Security: IEEE 802.1x Extensible Authentication Protocol (EAP)/Transport Layer Security (TLS)

    - Data Security

      - The fixed network camera offers a built-in secure element with Trusted Platform Module (TPM) that supports RSA 4096 bit for device identity and AES/DES/PKI encryption.

      - The fixed network camera offers advanced handling certificates, including:

        - Pre-installation of manufacturer origin device certificate

        - Automatic generation of self-signed unique certificates when required

      - The fixed network camera supports full end-to-end encryption when used with supported VMS.

      - The fixed network camera supports different encryption protocols, such as TLS 1.2, AES 256, AES 128, and TLS 1.3.

      - The fixed network camera supports multiple video authentication methods, such as MD5, SHA-1, SHA-256, and Checksum verification.

      - The fixed network camera offers XTS-AES for local storage encryption.

      - The fixed network camera offers signed firmware, secure boot and software sealing functionalities to ensure firmware protection.

      - The fixed network camera shall be compliant with IEC62443-4-1 and UL 2900-2-3.

  - ELECTRICAL

    - Power Input:

      - The fixed network camera shall support the following redundant power options:

        - 24 VAC

        - 12 to 26 VDC

        - PoE+ (IEEE 802.3at Type 2, Class 4)

    - Power Consumption (maximum)

      - The fixed network camera shall support the following maximum power consumption options:

        - VAC: 20.7 W

        - VDC: 28.4 W

        - PoE+: 25.5 W

  - MECHANICAL AND ENVIRONMENTAL

    - Material:

      - The fixed network camera housing shall be an anodized cast aluminum, rugged design, with a polycarbonate bubble.

    - Finish:

      - The fixed network camera shall offer a white finish.

    - Impact Resistance:

      - The fixed network camera shall offer IK11 impact resistance housing and bubble.

    - Dimensions (Ø x H):

      - The fixed network camera shall have 175mm x 148 mm (6.9 in. x 5.7 in.)

    - Temperature:

      - The fixed network camera shall support an operating temperature range of -50°C to 60°C (-58°F to 140°F).

      - The fixed network camera shall support a storage temperature range of -30°C to 70ºC (-22°F to 158ºF).

    - Relative Humidity (non-condensing):

      - The fixed network camera shall support an operating relative humidity of 	5% to 93%.

      - The fixed network camera shall support a storage relative humidity of 0 to 100%.

    - Ingress Protection:

      - The fixed network camera shall offer IP66/IP67 environmental protection.

      - The fixed network camera shall offer IP6K9K protection against foreign objects, water, and access.

    - Mounting:

      - The fixed network camera shall offer accessories to enable the mounting of the dome camera in the following circumstances:

        - Wall/ceiling mount

        - Pendant arm mount

        - Pipe mount

        - Pole mount

        - Corner mount

  - ACCESSORIES

    - Bubbles

      - NDA-8000-CBL		Clear replacement bubble

      - NDA-8000-TBL		Tinted Bubble

      - NDA-8000-HCBL	Hydrophilic bubble

    - Other Accessories

      - NDA-8000-WPIR	On-camera weather protector IR

      - NDA-8000-PIPWIR	Pendant interface plate, outdoor IR

      - NDA-8002-PLEN	In-ceiling plenum-rated mount kit, IR

      - NDA-8002-IC		In-ceiling mount kit, IR

      - NDA-8000-PCIR	Paintable cover, 4 pcs, IR

      - NDA-U-WMT		Pendant wall mount

      - NDA-U-PMT 		Pendant pipe mount, 12” (31cm)

      - NDA-U-PMTE		Pendant pipe extension, 20” (50cm)

      - NDA-U-PSMB		Pendant wall/ceiling mount SMB

      - NDA-U-PMAS 		Pole mount adapter small

      - NDA-U-PMAL 		Pole mount adapter large

      - NDA-U-RMT		Pendant parapet mount

      - NDA-U-CMT		Corner mount adapter

      - NDA-U-PA0		Surveillance cabinet 24VAC

      - NDA-U-PA1		Surveillance cabinet 120VAC

      - NDA-U-PA2		Surveillance cabinet 230VAC

      - NDA-U-WMP 		Wall mount plate

      - NDA-U-PMTS		Pendant pipe mount

      - NDA-U-DWMT		Dual pendant wall mount

      - LTC 9230/01		Parapet roof mount adapter

      - NBA-7070-PA0		Surveillance cabinet 24VAC

      - NBA-7070-PA1		Surveillance cabinet 120VAC

      - NBA-7070-PA2		Surveillance cabinet 230VAC

      - NDA-8000-CND             Conduit adapter FLEXIDOME 8100i 4pcs

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.

  - INSTALLATION

    - The contractor shall follow all Manufacturer-published guidance on proper installation and configuration of the installed cameras.

    - The Contractor shall test the system in conditions simulating the final installed environment

    - Reports:

      - System test results

      - System network addressing and device settings

  - STORAGE

    - Products shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION

202501310828